3D RECONSTRUCTION OF GERRARD HALL USING STRUCTURE-FROM-MOTION

Abstract

This project presents the implementation of a SfM pipeline for sparse 3D reconstruction of Gerrard Hall from a sequence of overlapping images. The objective is to understand and implement the fundamental stages involved in recovering camera motion and scene structure from multiple 2D images. The implemented pipeline includes SIFT feature extraction, feature matching, RANSAC-based outlier rejection, Fundamental Matrix estimation using the Eight-Point Algorithm, camera intrinsic estimation, Essential Matrix computation, recovery of relative camera rotation and translation, triangulation, and Perspective-n-Point (PnP)-based incremental camera registration.

A dataset containing 100 images was used for the experiment. The image resolution was approximately 1404 × 936 pixels. A working camera intrinsic matrix was constructed using focal lengths of 936 pixels and principal point coordinates of (702, 468). The initial two-view reconstruction produced 911 valid 3D landmarks. Additional images were incrementally registered using PnP, and new landmarks were generated through triangulation. The final automated reconstruction successfully registered 11 cameras and produced 5,165 sparse 3D landmarks.

The results demonstrate successful recovery of camera geometry and sparse scene structure. Representative camera registrations achieved mean reprojection errors of approximately 0.88–1.41 pixels. However, 89 of the 100 input images were not registered by the current implementation. Therefore, the final result is a sparse partial reconstruction rather than a complete 3D model. The project provides a functional foundation for further improvements such as global feature tracking, bundle adjustment, dense reconstruction, mesh generation, and texture mapping.

Keywords: Structure-from-Motion, SIFT, RANSAC, Fundamental Matrix, Eight-Point Algorithm, Essential Matrix, PnP, Triangulation, 3D Reconstruction.

1. INTRODUCTION

Three-dimensional reconstruction from multiple images is an important computer vision problem with applications in robotics, photogrammetry, augmented reality, cultural heritage preservation, and autonomous systems. A single image does not directly provide depth information. However, when multiple images of the same scene are captured from different viewpoints, geometric relationships between corresponding image points can be exploited to estimate both camera motion and 3D scene structure.

Structure-from-Motion (SfM) is a computational framework that performs this reconstruction from a collection of images. The basic objective is to determine the relative camera poses and recover a sparse set of 3D points that explain the observed image measurements.

In this project, an SfM pipeline was developed to reconstruct the structure of Gerrard Hall from 100 overlapping images. Instead of using a complete commercial or end-to-end reconstruction package, the major mathematical and computational stages of SfM were implemented and validated individually.

The overall workflow is:

Image acquisition → SIFT features → Feature matching → RANSAC → Fundamental Matrix → Eight-Point Algorithm → Camera calibration → Essential Matrix → Recover camera pose → Triangulation → PnP → Incremental reconstruction.

The project therefore provides practical understanding of the relationship between epipolar geometry, camera geometry, feature correspondence, and 3D reconstruction.

2. METHODOLOGY

2.1 Dataset and Camera Model

The experiment used 100 images of Gerrard Hall. The working images had an image size of approximately 1404 × 936 pixels.

The camera intrinsic matrix was represented as

K=fx0cx0fycy001.

The working parameters were

fx=fy=936,
cx=702, cy=468.

Thus,

K=93607020936468001.

The focal-length information available for the images was 24 mm. Since SfM reconstruction from a monocular sequence has an inherent scale ambiguity, the reconstruction coordinates represent a relative coordinate system rather than absolute physical dimensions.

2.2 SIFT Feature Extraction

SIFT was used to detect distinctive image features and generate 128-dimensional descriptors. SIFT was selected because its features are relatively robust to changes in scale, rotation, and viewpoint.

Representative results are shown in Table 1.

Table 1. SIFT feature extraction results.

| Image | Keypoints | Descriptor Dimension |
| --- | --- | --- |
| IMG_2331.JPG | 9,292 | 9292 × 128 |
| IMG_2332.JPG | 9,839 | 9839 × 128 |
| IMG_2333.JPG | 11,138 | 11138 × 128 |
| IMG_2334.JPG | 12,155 | 12155 × 128 |

The large number of detected features provided sufficient candidate correspondences for estimating the geometric relationships between images.

2.3 Feature Matching and RANSAC

Feature descriptors between overlapping images were matched using nearest-neighbour matching. A ratio test was used to remove ambiguous matches.

Since raw feature matching inevitably produces incorrect correspondences, RANSAC was used to identify geometrically consistent inliers. These inliers were subsequently used for Fundamental Matrix estimation.

The feature-matching process can be summarized as:

SIFT descriptors → Descriptor matching → Ratio test → RANSAC → Geometrically consistent correspondences.

2.4 Fundamental Matrix and Eight-Point Algorithm

For corresponding points xand x'in two images, the epipolar constraint is

x'TFx=0,

where Fis the Fundamental Matrix.

The Fundamental Matrix estimated for the initial image pair was:

F=-1.0434×10-81.9783×10-72.6435×10-59.1444×10-8-6.0172×10-8-3.9209×10-3-1.8830×10-43.7958×10-3-2.0242×10-2.

The calculated rank was

rank⁡F=2.

This satisfies the required rank-2 property of the Fundamental Matrix and provides an important validation of the implementation.

2.5 Essential Matrix and Camera Pose

The Essential Matrix was obtained using the camera intrinsic matrix:

E=KTFK.

Its singular value decomposition initially produced approximately

3.6821  3.6220  0.

The Essential Matrix was corrected by enforcing equal non-zero singular values. The corrected singular values were approximately

3.6520  3.6520  0.

This satisfies the required structure of a valid Essential Matrix.

Decomposition of the Essential Matrix produces multiple possible camera configurations. The correct configuration was selected using the cheirality condition, which requires reconstructed points to have positive depth in both cameras.

For the selected solution:

911/911

points had positive depth in both cameras.

2.6 Initial Triangulation

Triangulation was used to recover 3D coordinates from corresponding image points.

The initial reconstruction contained:

911 3D landmarks.

The reconstructed coordinate ranges were:

| Coordinate | Minimum | Maximum |
| --- | --- | --- |
| X | -9.645 | 6.734 |
| Y | -2.817 | 2.954 |
| Z | 6.383 | 16.922 |

Table 2. Initial 3D reconstruction statistics.

The first two camera centers were

C1=000

and

C2=0.9990.0400.021.

The baseline was normalized to approximately 1 reconstruction unit.

3. INCREMENTAL MULTI-VIEW RECONSTRUCTION

After establishing the initial two-view reconstruction, additional images were registered incrementally. For each new image, existing 3D landmarks were matched against image features. These 3D–2D correspondences were supplied to a RANSAC-based PnP procedure to estimate the new camera pose.

Once a camera was registered, new feature correspondences between the newly registered image and an existing image were triangulated to expand the 3D point cloud.

The incremental process was therefore:

Existing 3D landmarks → 3D–2D correspondences → PnP → New camera pose → Triangulation → New 3D landmarks.

3.1 Camera Registration Results

Several cameras were successfully registered with good geometric consistency.

Table 3. Representative PnP camera-registration results.

| Image | Correspondences | Inliers | Inlier Ratio | Mean Reprojection Error |
| --- | --- | --- | --- | --- |
| Image 3 | 410 | 406 | 99.02% | 0.882 px |
| Image 4 | 213 | 176 | 82.63% | 1.406 px |
| Image 5 | 135 | 114 | 84.44% | 1.395 px |
| Image 6 | 62 | 59 | 95.16% | 1.106 px |

The Image-6 registration initially failed when a larger and less reliable set of landmarks was used. A diagnostic analysis showed that the original high-quality landmarks produced a much stronger correspondence set. Using 62 reliable correspondences resulted in 59 PnP inliers and a mean reprojection error of 1.106 pixels.

This demonstrates the importance of reliable feature tracks in incremental SfM.

3.2 New Point Triangulation

Following registration of Image 5, new correspondences were triangulated.

The results were:

- Potential new matches: 877

- Positive-depth points: 855

- Accepted triangulated points: 764

- Previous landmarks: 911

- Updated landmarks: 1,675

For the accepted points, the reprojection errors were:

| Camera | Mean | Median | Maximum |
| --- | --- | --- | --- |
| Camera 4 | 1.073 px | 0.814 px | 2.993 px |
| Camera 5 | 1.046 px | 0.806 px | 2.925 px |

Table 4. Validation of newly triangulated landmarks.

The approximately 1-pixel mean reprojection errors indicate that the accepted triangulated points were reasonably consistent with the estimated camera poses.

4. FINAL SfM RESULTS

The automated incremental SfM procedure was subsequently executed over the complete set of 100 input images.

The final result was:

Table 5. Final SfM reconstruction statistics.

| Parameter | Result |
| --- | --- |
| Total input images | 100 |
| Registered cameras | 11 |
| Skipped images | 89 |
| Final 3D landmarks | 5,165 |

The recovered camera centers were:

| Camera | X | Y | Z |
| --- | --- | --- | --- |
| C1 | 0.000 | 0.000 | 0.000 |
| C2 | 0.999 | 0.040 | 0.021 |
| C3 | 1.942 | -0.036 | 0.007 |
| C4 | 2.653 | -0.095 | -0.017 |
| C5 | 3.427 | -0.019 | 0.041 |
| C6 | 4.710 | 0.124 | 0.453 |
| C7 | 5.823 | 0.180 | 0.825 |
| C8 | 6.851 | 0.396 | 1.458 |
| C9 | 7.325 | 0.562 | 1.959 |
| C10 | 7.669 | 0.642 | 2.297 |
| C12 | 8.139 | 5.630 | 2.937 |

Table 6. Estimated camera trajectory.

The camera centers generally show a forward progression through the scene, indicating that the SfM system successfully estimated camera motion for the registered views.

Figure 1. Estimated camera trajectory and registered camera centers.

Figure 2. Sparse 3D point-cloud reconstruction of Gerrard Hall.

The figures should be generated from the saved reconstruction data and inserted at this point in the report.

5. DISCUSSION

The experimental results demonstrate that the implemented pipeline successfully recovered important elements of multi-view geometry.

The Fundamental Matrix satisfied the required rank-2 constraint. The Essential Matrix was corrected to have two equal non-zero singular values and a near-zero third singular value. The cheirality test successfully identified a valid camera configuration, with all 911 initial reconstructed points having positive depth in both cameras.

The incremental reconstruction also demonstrated successful camera registration using PnP. The registered cameras achieved relatively low reprojection errors, with representative values between approximately 0.88 and 1.41 pixels.

The number of reconstructed landmarks increased from 911 initially to 5,165 in the final automated reconstruction. This indicates that the incremental process was capable of expanding the sparse scene model as additional cameras were registered.

However, only 11 of the 100 images were successfully registered. Therefore, the reconstruction is incomplete and should be described as a sparse partial reconstruction. The main limitation is the inability of the current implementation to establish sufficiently reliable 3D–2D correspondences for many of the remaining images.

A more robust production SfM system would maintain global feature tracks across multiple images rather than relying heavily on sequential image-to-image registration. It would also use image-pair selection based on overlap and correspondence quality, followed by bundle adjustment to jointly optimize camera poses and 3D landmarks.

6. LIMITATIONS AND FUTURE WORK

The current implementation has several limitations.

First, the camera intrinsic parameters were approximated from available focal-length and image-size information. A dedicated camera calibration procedure would provide more accurate intrinsic and lens-distortion parameters.

Second, the reconstruction has monocular scale ambiguity. Consequently, the baseline of approximately one unit is a normalized reconstruction scale rather than a physical measurement.

Third, only 11 of the 100 images were registered. Improving feature-track management and selecting the next image based on geometric overlap could significantly increase the number of registered cameras.

Fourth, bundle adjustment has not yet been incorporated. Bundle adjustment would jointly optimize the camera parameters and 3D landmark positions and could reduce accumulated reprojection error.

Finally, the current result is a sparse point cloud. Future work should include:

- Global multi-view feature tracking.

- Improved image-pair selection.

- Re-registration of skipped images.

- Bundle adjustment.

- Dense Multi-View Stereo.

- Dense point-cloud generation.

- Surface mesh reconstruction.

- Texture mapping.

These extensions would transform the current sparse reconstruction into a more complete and visually detailed 3D model.

7. CONCLUSION

This project implemented and evaluated a classical monocular Structure-from-Motion pipeline for sparse reconstruction of Gerrard Hall. The implementation covered the major stages of SfM, including SIFT feature extraction, feature matching, RANSAC, Fundamental Matrix estimation, the Eight-Point Algorithm, Essential Matrix estimation, camera pose recovery, cheirality verification, triangulation, and PnP-based incremental camera registration.

The initial two-view reconstruction produced 911 valid 3D landmarks. Subsequent incremental processing expanded the reconstruction to 5,165 landmarks and successfully estimated 11 camera poses from the 100-image dataset. The successful camera registrations showed low reprojection errors, generally around 1–1.5 pixels, demonstrating that the recovered camera geometry and 3D points were reasonably consistent.

The main limitation was the registration of only 11 out of 100 images. Consequently, the final output should be considered a sparse partial reconstruction rather than a complete 3D model. Nevertheless, the project successfully demonstrates the mathematical and computational principles behind Structure-from-Motion and provides a strong foundation for further development.

REFERENCES

[1] D. G. Lowe, “Distinctive image features from scale-invariant keypoints,” International Journal of Computer Vision, vol. 60, no. 2, pp. 91–110, Nov. 2004.

[2] R. Hartley and A. Zisserman, Multiple View Geometry in Computer Vision, 2nd ed. Cambridge, U.K.: Cambridge University Press, 2004.

[3] R. I. Hartley, “In defense of the eight-point algorithm,” IEEE Transactions on Pattern Analysis and Machine Intelligence, vol. 19, no. 6, pp. 580–593, Jun. 1997.

[4] M. A. Fischler and R. C. Bolles, “Random sample consensus: A paradigm for model fitting with applications to image analysis and automated cartography,” Communications of the ACM, vol. 24, no. 6, pp. 381–395, Jun. 1981.

[5] Z. Zhang, “A flexible new technique for camera calibration,” IEEE Transactions on Pattern Analysis and Machine Intelligence, vol. 22, no. 11, pp. 1330–1334, Nov. 2000.

[6] B. Triggs, P. McLauchlan, R. Hartley, and A. Fitzgibbon, “Bundle adjustment—A modern synthesis,” in Proc. Int. Workshop on Vision Algorithms: Theory and Practice, Corfu, Greece, 1999, pp. 298–372.

[7] J. L. Schönberger and J.-M. Frahm, “Structure-from-motion revisited,” in Proc. IEEE Conf. Computer Vision and Pattern Recognition (CVPR), Las Vegas, NV, USA, 2016, pp. 4104–4112.

[8] R. Szeliski, Computer Vision: Algorithms and Applications, 2nd ed. Cham, Switzerland: Springer, 2022.